P0.3 <- D5

P0.7 <- D9

P0.2 <- D4

P0.6 <- D8

P0.1 <- D3

P0.5 <- D7

P0.0 <- D2

P0.4 <- D6

STUPHORN LAB, TDT/NEUROPIXELS RIG: 3CH SYRINGE PUMP PIN MAPPING

MEGA pins	PCI-6229 pins

D9		P0.7

D8		P0.6

D7		P0.5

D6		P0.4

D5		P0.3

D4		P0.2

D3		P0.1

D2		P0.0

============================================================================

## 3-Channel Syringe Pump Controller for Arduino Mega

============================================================================

- Controls 3 stepper motors with auto-reversal at 0-40mm boundaries.

- Sends JSON messages to Raspberry Pi monitor via USB serial.

Monitors 8-bit GPIO input from external device (TDT/Plexon system):

- Bits 0-3: Magnitude (0-15, representing 1-16 units)

- Bit 4: Unused (reserved)

- Bits 5-6: Pump selection (10=Pump1, 01=Pump2, 11=Pump3)

- Bit 7: Trigger (rising edge triggers delivery)

GPIO Pin Mapping (reading left to right, Pin 2 to Pin 9):

Position:  0  1  2  3  4  5  6  7

Pin:       2  3  4  5  6  7  8  9

└──────┬──────┘  └─┬─┘  └─ Trigger

Amount      Pump

Example encodings:

Pump 1, Amount 1:  00000101 (binary) = 0x05 (hex)

Pump 1, Amount 2:  10000101 (binary) = 0x85 (hex)

Pump 2, Amount 1:  00000011 (binary) = 0x03 (hex)

Pump 3, Amount 5:  00100111 (binary) = 0x27 (hex)

Serial Commands (all start with ! and end with \r\n):

!GetCurrentPosition [pump]\r\n        	- Get position of pump (1-3)

!SetCurrentPosition [pump] [pos]\r\n 	- Set position in mm

!GetUnitSize [pump]\r\n              	- Get unit size in µL

!SetUnitSize [pump] [size]\r\n        	- Set unit size in µL

!Translate [pump] [distance]\r\n      	- Move ±distance in mm

!ManualReward [pump]\r\n              	- Deliver one unit manually

!SetCalibration [pump] [ppm]\r\n      	- Set pulses per mm (default 945)

!Home [pump]\r\n                      	- Move to position 0

!ResetCounter [pump]\r\n             	- Reset trigger counter

!GetStatus\r\n                       		 - Get full system status

Hardware:

- Input pins: 2-9 (GPIO bits 0-7)

- Pump 1: EN=10, STEP=11, DIR=12

- Pump 2: EN=13, STEP=14, DIR=15

- Pump 3: EN=16, STEP=17, DIR=18

- LED: Pin 13 (built-in)

MEGA pins	CB-68LPR		PCI-6229 pins

D9		J48		P0.7

D8		J16		P0.6

D7		J51		P0.5

D6		J19		P0.4

D5		J47		P0.3

D4		J49		P0.2

D3		J17		P0.1

D2		J52		P0.0